- Raspberry Pi 4 – Pin 2 (5V) connected to +ve on breadboard. This gives power to the whole board.

- Raspberry Pi 4 – Pin 6 (GND) connected to GND on breadboard.

- Ultrasonic Sensor – VCC connected to +ve and GND is connected to GND on breadboard.

- Plug TRIG into a blank rail, and plug that rail into GPIO 23 [Pin 16].

- Plug ECHO into a blank rail, link another blank rail using R1 (1kΩ resistor)

- Link your R1 rail with the GND rail using R2 (2kΩ resistor). Leave a space between the two resistors.

- https://thepihut.com/blogs/raspberry-pi-tutorials/hc-sr04-ultrasonic-range-sensor-on-the-raspberry-pi

- Add GPIO 24 [Pin 18] to the rail with your R1 (1kΩ resistor). This GPIO pin needs to sit between R1 and R2

- Raspberry Pi is now connected to Ultrasonic sensor.

- Put the Buzzer on the bread board and connect GPIO 17 [Pin 11] to the +ve end of buzzer and the negative end of the buzzer is connected to the GND on the breadboard.

- Buzzer is now connected to the Pi.

- Connected a 470-ohm resistor to the breadboard. Connect GPIO 18 [Pin 12] to the resistor on one end and connect the positive end of the LED to the other end of the resistor.

- Connect one end of a wire to the GND rail on the breadboard and the other end should be in the same row as the LED.

- All connections are done.